http://www.acnc.com

2004.03.15

| RAID 0: Striped Disk Array without Fault Tolerance RAID Level 0 requires a minimum of 2 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages RAID 0 implements a striped disk array, the data is broken down into blocks and each block is written to a separate disk drive I/O performance is greatly improved by spreading the I/O load across many channels and drives Best performance is achieved when data is striped across multiple controllers with only one drive per controller No parity calculation overhead is involved Very simple design Easy to implement |  |  |  | Disadvantages Not a "True" RAID because it is NOT fault-tolerant The failure of just one drive will result in all data in an array being lost Should never be used in mission critical environments Recommended Applications - Video Production and Editing - Image Editing - Pre-Press Applications - Any application requiring high bandwidth |
| --- | --- | --- | --- | --- | --- |

| RAID 1: Mirroring and Duplexing For Highest performance, the controller must be able to perform two concurrent separate Reads per mirrored pair or two duplicate Writes per mirrored pair. RAID Level 1 requires a minimum of 2 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages One Write or two Reads possible per mirrored pair Twice the Read transaction rate of single disks, same Write transaction rate as single disks 100% redundancy of data means no rebuild is necessary in case of a disk failure, just a copy to the replacement disk Transfer rate per block is equal to that of a single disk Under certain circumstances, RAID 1 can sustain multiple simultaneous drive failures Simplest RAID storage subsystem design |  |  |  | Disadvantages Highest disk overhead of all RAID types (100%) - inefficient Typically the RAID function is done by system software, loading the CPU/Server and possibly degrading throughput at high activity levels. Hardware implementation is strongly recommended May not support hot swap of failed disk when implemented in "software" Recommended Applications - Accounting - Payroll - Financial - Any application requiring very high availability |  |
| --- | --- | --- | --- | --- | --- | --- |
| RAID 2: Hamming Code ECC Each bit of data word is written to a data disk drive (4 in this example: 0 to 3). Each data word has its Hamming Code ECC word recorded on the ECC disks. On Read, the ECC code verifies correct data or corrects single disk errors. |  |  |  |  |  |  |

|  | Characteristics/Advantages "On the fly" data error correction Extremely high data transfer rates possible The higher the data transfer rate required, the better the ratio of data disks to ECC disks Relatively simple controller design compared to RAID levels 3,4 & 5 |  |  |  | Disadvantages Very high ratio of ECC disks to data disks with smaller word sizes - inefficient Entry level cost very high - requires very high transfer rate requirement to justify Transaction rate is equal to that of a single disk at best (with spindle synchronization) No commercial implementations exist / not commercially viable |
| --- | --- | --- | --- | --- | --- |

| RAID 3: Parallel transfer with parity The data block is subdivided ("striped") and written on the data disks. Stripe parity is generated on Writes, recorded on the parity disk and checked on Reads. RAID Level 3 requires a minimum of 3 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages Very high Read data transfer rate Very high Write data transfer rate Disk failure has an insignificant impact on throughput Low ratio of ECC (Parity) disks to data disks means high efficiency |  |  |  | Disadvantages Transaction rate equal to that of a single disk drive at best (if spindles are synchronized) Controller design is fairly complex Very difficult and resource intensive to do as a "software" RAID Recommended Applications - Video Production and live streaming - Image Editing - Video Editing - Prepress Applications - Any application requiring high throughput |
| --- | --- | --- | --- | --- | --- |

| RAID 4: Independent Data disks with shared Parity disk Each entire block is written onto a data disk. Parity for same rank blocks is generated on Writes, recorded on the parity disk and checked on Reads. RAID Level 4 requires a minimum of 3 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages Very high Read data transaction rate Low ratio of ECC (Parity) disks to data disks means high efficiency High aggregate Read transfer rate |  |  |  | Disadvantages Quite complex controller design Worst Write transaction rate and Write aggregate transfer rate Difficult and inefficient data rebuild in the event of disk failure Block Read transfer rate equal to that of a single disk |
| --- | --- | --- | --- | --- | --- |

| RAID 5: Independent Data disks with distributed parity blocks Each entire data block is written on a data disk; parity for blocks in the same rank is generated on Writes, recorded in a distributed location and checked on Reads. RAID Level 5 requires a minimum of 3 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages Highest Read data transaction rate Medium Write data transaction rate Low ratio of ECC (Parity) disks to data disks means high efficiency Good aggregate transfer rate |  |  |  | Disadvantages Disk failure has a medium impact on throughput Most complex controller design Difficult to rebuild in the event of a disk failure (as compared to RAID level 1) Individual block data transfer rate same as single disk Recommended Applications - File and Application servers - Database servers - WWW, E-mail, and News servers - Intranet servers - Most versatile RAID level |
| --- | --- | --- | --- | --- | --- |

| RAID 6: Independent Data disks with two independent distributed parity schemes |  |
| --- | --- |

|  | Characteristics/Advantages RAID 6 is essentially an extension of RAID level 5 which allows for additional fault tolerance by using a second independent distributed parity scheme (two-dimensional parity) Data is striped on a block level across a set of drives, just like in RAID 5, and a second set of parity is calculated and written across all the drives; RAID 6 provides for an extremely high data fault tolerance and can sustain multiple simultaneous drive failures Perfect solution for mission critical applications |  |  |  | Disadvantages More complex controller design Controller overhead to compute parity addresses is extremely high Write performance can be brought on par with RAID Level 5 by using a custom ASIC for computing Reed-Solomon parity Requires N+2 drives to implement because of two-dimensional parity scheme |
| --- | --- | --- | --- | --- | --- |

| RAID 7: Optimized Asynchrony for High I/O Rates as well as High Data Transfer Rates Architectural Features: - All I/O transfers are asynchronous, independently controlled and cached including host interface transfers - All reads and write are centrally cached via the high speed x-bus - Dedicated parity drive can be on any channel - Fully implemented process oriented real time operating system resident on embedded array control microprocessor - Embedded real time operating system controlled communications channel - Open system uses standard SCSI drives, standard PC buses, motherboards and memory SIMMs - High speed internal cache data transfer bus (X-bus) - Parity generation integrated into cache - Multiple attached drive devices can be declared hot standbys - Manageability: SNMP agent allows for remote monitoring and management |  |
| --- | --- |

|  | Characteristics/Advantages Overall write performance is 25% to 90% better than single spindle performance and 1.5 to 6 times better than other array levels Host interfaces are scalable for connectivity or increased host transfer bandwidth Small reads in multi user environment have very high cache hit rate resulting in near zero access times Write performance improves with an increase in the number of drives in the array Access times decrease with each increase in the number of actuators in the array No extra data transfers required for parity manipulation RAID 7 is a registered trademark of Storage Computer Corporation. |  |  |  | Disadvantages One vendor proprietary solution Extremely high cost per MB Very short warranty Not user serviceable Power supply must be UPS to prevent loss of cache data |
| --- | --- | --- | --- | --- | --- |

| RAID 10: Very High Reliability combined with High Performance RAID Level 10 requires a minimum of 4 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages RAID 10 is implemented as a striped array whose segments are RAID 1 arrays RAID 10 has the same fault tolerance as RAID level 1 RAID 10 has the same overhead for fault-tolerance as mirroring alone High I/O rates are achieved by striping RAID 1 segments Under certain circumstances, RAID 10 array can sustain multiple simultaneous drive failures Excellent solution for sites that would have otherwise gone with RAID 1 but need some additional performance boost |  |  |  | Disadvantages Very expensive / High overhead All drives must move in parallel to proper track lowering sustained performance Very limited scalability at a very high inherent cost Recommended Applications - Database server requiring high performance and fault tolerance |
| --- | --- | --- | --- | --- | --- |

| RAID 53: High I/O Rates and Data Transfer Performance RAID Level 53 requires a minimum of 5 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages RAID 53 should really be called "RAID 03" because it is implemented as a striped (RAID level 0) array whose segments are RAID 3 arrays RAID 53 has the same fault tolerance as RAID 3 as well as the same fault tolerance overhead High data transfer rates are achieved thanks to its RAID 3 array segments High I/O rates for small requests are achieved thanks to its RAID 0 striping Maybe a good solution for sites who would have otherwise gone with RAID 3 but need some additional performance boost |  |  |  | Disadvantages Very expensive to implement All disk spindles must be synchronized, which limits the choice of drives Byte striping results in poor utilization of formatted capacity |
| --- | --- | --- | --- | --- | --- |

| RAID 0+1: High Data Transfer Performance RAID Level 0+1 requires a minimum of 4 drives to implement |  |
| --- | --- |

|  | Characteristics/Advantages RAID 0+1 is implemented as a mirrored array whose segments are RAID 0 arrays RAID 0+1 has the same fault tolerance as RAID level 5 RAID 0+1 has the same overhead for fault-tolerance as mirroring alone High I/O rates are achieved thanks to multiple stripe segments Excellent solution for sites that need high performance but are not concerned with achieving maximum reliability |  |  |  | Disadvantages RAID 0+1 is NOT to be confused with RAID 10. A single drive failure will cause the whole array to become, in essence, a RAID Level 0 array Very expensive / High overhead All drives must move in parallel to proper track lowering sustained performance Very limited scalability at a very high inherent cost Recommended Applications - Imaging applications - General fileserver |
| --- | --- | --- | --- | --- | --- |